Flood Risk of Domestic Properties in Mill Street, Elsing, Dereham (December 2025)

The purpose of assembling this data is for Elsing Parish Council to ascertain, through consultation with the Water Management Alliance, Kings Lynn, whether any practical steps may be taken to reduce the flood risk to certain residents from the River Wensum.

Details (in metres) of the properties most at risk are as follows:

| HOUSE NAME | POSTCODE | ALTITUDE (HOUSE) | ALTITUDE (R WENSUM SURFACE) | NOTE |  |
| --- | --- | --- | --- | --- | --- |
| Mill Farm Annexe* | NR20 3EJ | 21 | 18 | River Wensum water level varies, but 0.6 metres is average. The highest recent peak was 1.1 metres in 1993. Levels on the map used here are dated 18 /4 / 25. Is this average? Probably. There are no figures for April, the date of the map. But for 2025 there was ‘normal’ flooding in January, and by March the level was 70% of average. So the figures in the column on the left are probably not unusual. |  |
| Elsing Mill | NE20 3EJ | 20 | 17 |  |  |
| Marsh Cottage (not on map) | NR20 4JS | 19 | 17 |  |  |
| Willow Mead | NR20 3EH | 19 | 17 |  |  |
| Enfin | NR20 3EH | 20 | 17 |  |  |
| Coppers, Watermeadow | NR20 3EJ | 20 | 17 |  |  |
| Brook House, Tu Bay Cottage | NR20 3EJ | 19 | 17 |  |  |
| Chestnut Cottage | NR20 3EJ | 18 | 17 |  |  |
| Three Bridges Farmhouse | NR20 3EW | 18 | 17 |  |  |

Sources: Google Earth (measurements only to the nearest metre), GOV..UK

The GOV.UK flood risk summary for this area is ‘low’. However we know that in these days of climate change, the past is not always an indicator of the future, and we are constantly advised to expect more extreme weather events. Far more information is therefore needed than the data above, which only relates to specific properties (who could be advised to have sandbags, air-brick covers, etc at the ready), but flooding is also determined by topography far beyond their location. Normally, floodwater in this area has first to fill its floodplain before there is any threat to property. But has this floodplain been reconfigured in any way, for example by the NOWZ excavations for cable pipelines which cross the Wensum at Elsing? Sluicegates at Elsing Mill moderate the flow of floodwater up to a point. The rate at which floodwater can be carried away is fixed by the weir at Lyng.  Flooding sometimes defies logic – the first property above* has seen floodwater 0.3 m beneath the floorboards, while lower-lying properties were not flooded.  It seems floodwater can sometimes be better contained by proper maintenance of ditches, drainage channels, buffer strips, leaky dams, bunds, etc, as signalled in the May 2024 report by Breckland Council and CPRE Norfolk. The equation which brings together all the above factors cannot be correctly understood by this Parish Council. Thus we would be pleased to hear from the Water Management Alliance or a person proposed by them, who would be better qualified to do so.

Three of the four local landowners were contacted but were not able to respond to our question “If there were a greater depth of floodwater than there has been before, where would it go?” However they were agreed on the following:

1. Floodwater is slower to pass than need be because the authorities no longer clear weed growth.

2. The nutrients which produce weed growth are due to waste from Bylaugh Sewage Works and not from nitrates, which are not used on adjacent meadows.

3. The properties most at risk are Marsh Cottage and Three Bridges farmhouse.

pto...

Meeting of Cllrs Chaplin, Horncastle and Laws with Mr Tom Hunter of the Kings Lynn Water Management Alliance,  5 / 3 / 26

Mr Hunter was able to provide the perspective asked for in the first page of this document. In his view, the government assessment of “Low” flood risk is appropriate for Elsing. Below are listed the additional points of interest which were discussed.

* Mr Hunter provided three useful maps (A,B,C), on which you might like to try and locate your own house:

A  shows where floodwater is likely to go. Dark blue represents lakes and the river. Mid blue shows the extent of flooding in a bad year, but which we are familiar with.

Light or very light blue shows where, in theory, unusually high floodwater would reach and includes all the above properties; but this would be a one-in-a-thousand year

event.

B  shows, in blue, the important drainage ditches which feed water into the Wensum. In the past they may have also served as mill leats. Property owners either side of

these share responsibility for removing blockages of any sort. They may do so without a permit but must have regard to wildlife, breeding seasons, etc. Any building work,

driveways, etc which might reduce flow should be reported.

C  shows, with the same colour-coding as in A, the possible extent of surface water. This is rainfall which has naturally collected along the drainage ditches, at a slightly

higher level than the flooded river, but which may or may not flow away fast enough. This accounts for the flooding at the first property above but not elsewhere.

You can find these maps at     https://check-long-term-flood-risk.service.gov.uk/map

* The Internal Drainage Board is responsible for drainage ditches, and the Environment Agency covers the river as a whole. The EA does not allow landowners to use machinery to dredge or clear weeds, fearing disturbance of the delicate ecological balance. In some cases permits may be granted, but these are expensive and time-consuming. Fallen trees may usually be dealt with by the landowner, as may any clearing of weeds done by hand.

* Enquiries regarding the discharge from the sewage works at Bylaugh may be addressed to the E A Water Quality Team, Dragonfly House, 2 Gilders Way, Norwich.

Elsing Parish Council, Marsh 2026